EXP 9

CODE:

clc;

clear;

close all;

% Experiment 9: AM Generation and Detection under Noisy Conditions

% Parameters

fs = 100000;

Am = 1;

fm = 1000;

Ac = 2;

fc = 10000;

mu = 0.8;

SNR_dB = 15;

% Time vector

t = 0:1/fs:0.01;

% Generate message signal

m = Am*cos(2*pi*fm*t);

% Generate conventional AM signal

am_signal = Ac*(1 + mu*m/Am).*cos(2*pi*fc*t);

% Add AWGN to the AM signal

signal_power = mean(am_signal.^2);

noise_power = signal_power/(10^(SNR_dB/10));

noise = sqrt(noise_power)*randn(size(am_signal));

noisy_signal = am_signal + noise;

% Envelope detection

envelope = abs(hilbert(noisy_signal));

% Remove carrier DC component and recover message

recovered = (envelope - Ac)/mu;

% Plot results

figure('Name','Experiment 9 - AM under Noisy Conditions');

subplot(4,1,1);

plot(t,m);

title('Original Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,2);

plot(t,am_signal);

title('Transmitted AM Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,3);

plot(t,noisy_signal);

title('AM Signal with AWGN Noise');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,4);

plot(t,m,'b',t,recovered,'r');

title('Original and Recovered Message Signals');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Original','Recovered');

grid on;

% Display results

fprintf('Experiment 9 Results\n');

fprintf('Message frequency = %.2f Hz\n',fm);

fprintf('Carrier frequency = %.2f Hz\n',fc);

fprintf('AM modulation index = %.2f\n',mu);

fprintf('Specified SNR = %.2f dB\n',SNR_dB);

fprintf('Noise power = %.6f\n',noise_power);

fprintf('AM signal generated and envelope detection completed.\n');

OUTPUT: